15A NCAC 02T .0606	SETBACKS

(a)  The setbacks for irrigation sites shall be as follows:

Spray	Drip

(feet)	(feet)

Each habitable residence or place of assembly under separate ownership

or not to be maintained as part of the project site	400	100

Each habitable residence or place of assembly owned by the

permittee to be maintained as part of the project site	200	15

Each private or public water supply source	100	100

Surface waters such as intermittent and perennial streams, perennial waterbodies, and

wetlands	100	100

Groundwater lowering ditches where the bottom of the ditch intersects the SHWT	100	100

Surface water diversions such as ephemeral streams, waterways, and ditches	25	25

Each well with exception of monitoring wells	100	100

Each property line	150	50

Top of slope of embankments or cuts of two feet or more in vertical height	15	15

Each water line from a disposal system	10	10

Subsurface groundwater lowering drainage systems	100	100

Public right of way	50	50

Nitrification field	20	20

Each building foundation or basement	15	15

(b)  Treatment and storage facilities associated with systems permitted under this Section shall adhere to the setback requirements in Section .0500 of this Subchapter except as provided in this Rule.

(c)  Setback waivers shall be written, notarized, signed by all parties involved, and recorded with the county Register of Deeds. Waivers involving the compliance boundary shall be in accordance with 15A NCAC 02L .0107.

(d)  Setbacks to property lines established in Paragraphs (a) and (b) of this Rule shall not be applicable if the permittee, or the entity from which the permittee is leasing, owns both parcels separated by the property line.

(e)  Habitable residences or places of assembly under separate ownership constructed after the non-discharge facilities were originally permitted or subsequently modified are exempt from the setback requirements in Paragraphs (a) and (b) of this Rule.

History Note:	Authority G.S. 143-215.1; 143-215.3(a);

Eff. September 1, 2006;

Readopted Eff. September 1, 2018.